PyLTSpice Workshop

Automating LTSpice Simulations with Python

# Overview

This workshop teaches you how to control LTSpice from Python using the PyLTSpice library. Instead of manually adjusting component values and re-running simulations through the GUI, you will write scripts that automate the entire process — from editing schematics to parsing results and generating plots.

By the end of the workshop you will be able to:

- Run LTSpice simulations headlessly from a Python script

- Sweep component values across multiple combinations automatically

- Extract and plot simulation data using Matplotlib

- Perform Monte Carlo tolerance analysis to understand real-world component variation

# Prerequisites

## Software

| Software | Notes |
| --- | --- |
| LTSpice | Latest version from Analog Devices (ADI). Free download at analog.com/ltspice |
| Python 3.10+ | Download at python.org. Python 3.13 confirmed working. |
| VS Code | Recommended editor. Any editor works. |
| pip | Included with Python. Used to install PyLTSpice and plotting libraries. |

## Python Packages

Install all required packages before the workshop with:

pip install PyLTSpice matplotlib numpy

## Knowledge

- Basic Python — variables, functions, loops, imports

- Basic circuit theory — RC filters, Bode plots, cutoff frequency

- Familiarity with LTSpice GUI (drawing schematics, running .ac simulations)

# Setup

## Step 1 — Build the Base Schematic

Before running any scripts, you need an LTSpice schematic (.asc file) with parameterized component values. The scripts modify these parameters for each simulation run.

Draw a simple RC low-pass filter in LTSpice with the following settings:

| Element | Value / Setting |
| --- | --- |
| V1 (voltage source) | Set AC amplitude to 1 (AC 1) |
| R1 (resistor) | Set value to {R} — curly braces required |
| C1 (capacitor) | Set value to {C} — curly braces required |
| .param directive | Add .param R=15k C=6.8n to the canvas |
| .ac directive | Add .ac dec 100 10 1000000 to the canvas |

Why curly braces? In LTSpice schematics, {R} tells LTSpice to look up the value of parameter R from a .param directive. Without curly braces, LTSpice treats the letter R as an unknown and the simulation fails. In raw netlists (.net files), no curly braces are needed — plain R and C work directly.

## Step 2 — Update Paths in the Script

Open pyltspice_workshop.py and update the three path constants at the top of the file:

LTSPICE_PATH = r'C:\Program Files\ADI\LTspice\LTspice.exe' SCHEMATIC_PATH = r'C:\Users\YourName\Documents\LTspice\YourSchematic.asc' OUTPUT_FOLDER = r'C:\Users\YourName\Downloads\rc_sweep_output'

To verify your LTSpice path is correct, run the following in PowerShell:

Test-Path "C:\Program Files\ADI\LTspice\LTspice.exe"

This should return True. If it returns False, find the correct path and update the script.

## Step 3 — Run the Script

python pyltspice_workshop.py

The script runs all three use cases in sequence. Output files (plots and schematics) are saved to OUTPUT_FOLDER.

# The Three Use Cases

## Use Case 1 — Parameter Sweep

Run the same circuit with different R and C values and overlay all the Bode plots for direct comparison. This is the most common use of PyLTSpice and a natural extension of what you already do manually in LTSpice.

What it does:

- Defines a list of R values and a list of C values

- Generates every R+C combination (e.g. 3 R values x 3 C values = 9 simulations)

- Runs each simulation, reads the .raw output, and plots magnitude and phase

- Color-codes each curve and marks the theoretical -3 dB cutoff with a dashed vertical line

Key concept — the combination loop:

combinations = [(R, C) for R in R_VALUES for C in C_VALUES] # Creates every pair, e.g. [(15k, 6.8n), (15k, 15n), (15k, 22n), ...]

Output file: uc1_sweep.png

## Use Case 2 — AC Analysis & Custom Matplotlib Plotting

Run a single simulation and build a polished, annotated Bode plot. This use case focuses on what you can do with the data after the simulation runs — going well beyond LTSpice's built-in waveform viewer.

What it does:

- Simulates at the target design point (15 kΩ, 6.8 nF → fc ≈ 1560 Hz)

- Overlays the theoretical response curve alongside the simulated data

- Annotates the -3 dB point, cutoff frequency, and -20 dB/decade slope with arrows

- Marks -45° phase at fc as a sanity check

Key concept — complex-valued AC data:

vout = ltr.get_trace('V(out)').get_wave(0) # complex array mag_db = 20 * np.log10(np.abs(vout)) # magnitude in dB phase = np.angle(vout, deg=True) # phase in degrees

LTSpice stores AC voltages as complex numbers. The magnitude gives you gain, the angle gives you phase shift. NumPy handles both with one line each.

Output file: uc2_ac_analysis.png

## Use Case 3 — Monte Carlo / Tolerance Analysis

Real resistors and capacitors are not exactly their labeled value. A 15 kΩ resistor with ±5% tolerance could be anywhere from 14.25 kΩ to 15.75 kΩ. This use case simulates that variation across many random samples so you can see how manufacturing tolerances affect your circuit.

What it does:

- Randomly samples R and C values within their tolerance band (default ±5%)

- Runs a simulation for each sample (default 30 samples)

- Plots all curves as faint transparent lines with the nominal design in bold red

- Displays a statistics box with mean, standard deviation, min, and max cutoff frequency

- Generates a histogram of the cutoff frequency distribution as a bonus plot

Key concept — random sampling with NumPy:

R_samples = np.random.uniform( R_nom * (1 - R_tolerance), # lower bound R_nom * (1 + R_tolerance), # upper bound N_SAMPLES # number of draws )

What to look for in the Monte Carlo output: • If the spread of fc values is narrow, your design is robust to component variation. • If the spread is wide, consider using tighter tolerance components (±1% instead of ±5%). • The histogram shows whether the distribution is centered on your target fc.

Output files: uc3_monte_carlo.png, uc3_histogram.png

# Imports Reference

Every import in the script is listed below with a plain-English explanation of what it does.

| Import | What it does |
| --- | --- |
| numpy (np) | Math and arrays. Used for dB conversion, phase calculation, and random sampling. |
| matplotlib.pyplot (plt) | Creates figures, axes, and all plot elements — lines, labels, annotations. |
| matplotlib.cm (cm) | Provides color maps. tab10 assigns a unique color to each sweep curve. |
| PyLTSpice.LTspice | Runs LTSpice headlessly from Python via LTspice.run(). |
| PyLTSpice.RawRead | Reads and parses the binary .raw file LTSpice generates after each simulation. |
| PyLTSpice.SpiceEditor | Opens your .asc schematic and lets you modify .param values programmatically. |
| os | Creates the output folder and checks whether files exist. |
| shutil | Copies your schematic for each iteration so the original is never modified. |
| subprocess | Launches LTSpice as an external process to open the schematic after the sweep. |

# Output Files

All output is saved to OUTPUT_FOLDER. After the script runs you will find:

| File | Description |
| --- | --- |
| uc1_sweep.png | Overlaid Bode plots for all R/C combinations from Use Case 1 |
| uc2_ac_analysis.png | Annotated single-design Bode plot from Use Case 2 |
| uc3_monte_carlo.png | Monte Carlo Bode plot overlay from Use Case 3 |
| uc3_histogram.png | Histogram of cutoff frequency spread from Use Case 3 |
| sweep_N.asc | Modified schematic copies used for each Use Case 1 simulation |
| uc2_ac.asc | Modified schematic used for Use Case 2 |
| mc_N.asc | Modified schematic copies used for each Monte Carlo sample |
| *.raw | Binary simulation output files — readable by RawRead or LTSpice |

# Troubleshooting

| Error | Fix |
| --- | --- |
| FileNotFoundError on .asc path | The SCHEMATIC_PATH is wrong. Use PowerShell Get-ChildItem to find your .asc file. |
| IndexError: doesn't contain trace V(out) | Your node is named differently. Check valid trace names printed in the error — update the get_trace() call to match. |
| No .raw file found | LTSpice failed silently. Open the .asc file manually in LTSpice and check for netlist errors. |
| Could not open LTSpice | LTSPICE_PATH is wrong. Run Test-Path in PowerShell to verify the exe location. |
| LTSpice GUI opens instead of running | Some versions require -b (batch) flag. Wrap the run call with subprocess.run([LTSPICE_PATH, '-b', asc_path]). |
| {R} or {C} not found by SpiceEditor | Your .asc must have a .param R=... and .param C=... directive on the canvas. Add them in LTSpice via Edit > SPICE Directive. |

# Quick Reference

## RC Filter Cutoff Frequency

fc = 1 / (2 π R C) Example: R = 15 kΩ, C = 6.8 nF fc = 1 / (2 π × 15000 × 0.0000000068) ≈ 1560 Hz At fc: magnitude = -3 dB, phase = -45° Above fc: rolls off at -20 dB per decade

## Useful PyLTSpice Patterns

# Read all available trace names ltr = RawRead(raw_path) print([t.name for t in ltr._plots[0]._trace_info]) # Get frequency axis freq = np.array(ltr.get_trace('frequency').get_wave(0)).real # Get output voltage (complex) vout = np.array(ltr.get_trace('V(out)').get_wave(0)) # Convert to dB and degrees mag_db = 20 * np.log10(np.abs(vout)) phase = np.angle(vout, deg=True)